Parkinson’s Disease MRI Processing and ROI Signal Extraction

Project Description

This project implements an automated neuroimaging pipeline for analyzing functional MRI (fMRI) data from Parkinson’s disease patients and healthy control subjects. The goal is to preprocess resting-state BOLD images, normalize them into a standard brain space, and extract neural time-series signals from motor-related brain regions.

The pipeline integrates FSL neuroimaging tools with Python-based analysis to create a reproducible workflow for multi-subject MRI processing.

The workflow includes:

- Brain extraction from anatomical MRI

- Functional-to-structural image registration

- Structural-to-standard (MNI) normalization

- Nonlinear spatial transformation

- Atlas alignment

- ROI-based signal extraction

The extracted signals focus on motor network regions relevant to Parkinson’s disease, including:

- Primary Motor Cortex (M1)

- Supplementary Motor Area (SMA)

- Putamen

- Substantia Nigra

Data Source

The dataset used in this project is obtained from the Parkinson’s disease dataset provided by the 1000 Functional Connectomes Project (INDI).

Dataset link:  https://fcon_1000.projects.nitrc.org/indi/retro/parkinsons.html

This dataset contains resting-state fMRI scans and anatomical MRI scans from both Parkinson’s disease patients and healthy control participants.

Dataset Summary

Total participants used in this project:

- 27 Parkinson’s Disease subjects

- 16 Healthy Control subjects

Total:

43 participants

Each subject contains two main MRI modalities:

Structural MRI

anat/*T1w.nii.gz

Functional MRI

func/*bold.nii.gz

The functional MRI images are 4D BOLD images, meaning they contain three spatial dimensions and one time dimension.

Software and Tools

Neuroimaging Software

The pipeline uses FSL (FMRIB Software Library) for MRI preprocessing and spatial registration.

FSL documentation:  https://fsl.fmrib.ox.ac.uk/fsl/fslwiki

Main FSL tools used in this project:

| Tool | Purpose |
| --- | --- |
| BET | Brain extraction |
| FLIRT | Linear registration |
| FNIRT | Nonlinear spatial normalization |
| EPI_REG | Functional-to-structural alignment |
| APPLYWARP | Apply spatial transformations |

Python Libraries

The pipeline is automated using Python.

Libraries used:

- nibabel

- numpy

- pandas

- nilearn

- subprocess

- glob

- os

Purpose of the libraries:

- Nibabel – reading and writing NIfTI neuroimaging files

- NumPy – numerical computation and signal extraction

- Pandas – storing ROI time series

- Nilearn – atlas resampling and neuroimaging utilities

- Subprocess – running FSL commands from Python

Atlas for ROI Extraction

The project uses the AAL3 atlas for defining brain regions in standard MNI space.

Atlas file:

AAL3v1_1mm.nii.gz

Selected motor-related regions used for signal extraction:

| Region | Atlas Label |
| --- | --- |
| M1 Left | 1 |
| M1 Right | 2 |
| SMA Left | 15 |
| SMA Right | 16 |
| Putamen Left | 77 |
| Putamen Right | 78 |
| Substantia Nigra pars compacta Left | 161 |
| Substantia Nigra pars compacta Right | 162 |
| Substantia Nigra pars reticulata Left | 163 |

Each region is processed individually without merging atlas labels.

Dataset Preparation

The dataset is organized using a BIDS-like directory structure.

Example subject folder:

dataset/
 ├── sub-001
 │    ├── anat
 │    │     └── sub-001_T1w.nii.gz
 │    └── func
 │          └── sub-001_bold.nii.gz

Where:

- anat/ contains the T1 structural MRI

- func/ contains the functional BOLD MRI

The pipeline automatically searches these folders to locate the necessary input files.

Processing Workflow

The pipeline processes each subject sequentially using the following steps.

Figure: Workflow of the MRI preprocessing pipeline showing BOLD–T1 registration

Step 1: Brain Extraction

The structural T1 image is processed to remove non-brain tissues such as the skull and scalp.

FSL command used:

bet T1.nii.gz T1_brain.nii.gz -f 0.5

Output file:

T1_brain.nii.gz

Step 2: Mean BOLD Image Generation

The 4D BOLD image is converted into a 3D reference image by averaging across the time dimension.

FSL command:

fslmaths bold.nii.gz -Tmean mean_bold.nii.gz

Output file:

mean_bold.nii.gz

This image is used as the reference image for functional registration.

Step 3: Functional to Structural Registration

The mean BOLD image is aligned to the subject's anatomical T1 image using Boundary Based Registration (BBR).

FSL tool:

epi_reg

Output transformation matrix:

func2struct.mat

This matrix maps:

BOLD space → T1 anatomical space

Step 4: Structural to MNI Linear Registration

The brain-extracted T1 image is registered to the MNI152 standard brain template using linear registration.

FSL tool:

flirt

Output transformation matrix:

T1_to_MNI.mat

Step 5: Nonlinear Spatial Normalization

To improve alignment accuracy, nonlinear registration is performed using FNIRT.

FNIRT computes a deformation field that maps the subject's anatomical image to the MNI template.

Output file:

T1_to_MNI_warpcoef.nii.gz

Step 6: Apply Warp to BOLD Image

The functional BOLD image is transformed into MNI standard space using the previously calculated transformations.

FSL command:

applywarp

Transformations applied:

- BOLD → T1 (func2struct.mat)

- T1 → MNI (warpcoef)

Output file:

bold_in_MNI.nii.gz

Step 7: Atlas Resampling

The AAL3 atlas is resampled to match the resolution and voxel grid of the normalized BOLD image.

Python function used:

resample_to_img()

Output file:

AAL3_resampled.nii.gz

Step 8: ROI Signal Extraction

For each region defined in the atlas:

- A region mask is created.

- Voxels belonging to that region are identified.

- The mean BOLD signal across those voxels is calculated.

This produces a time-series signal for each brain region.

Output Files

For each subject, the pipeline produces the following outputs.

Processed Images:

T1_brain.nii.gz
mean_bold.nii.gz
bold_in_MNI.nii.gz
AAL3_resampled.nii.gz

Transformation Files:

func2struct.mat
T1_to_MNI.mat
T1_to_MNI_warpcoef.nii.gz

Extracted ROI Signals:

ROI_individual_timeseries.csv

Each column of the output corresponds to a specific brain region, and each row represents the time point of the BOLD signal.